80TRK.DOC

TRiPsoft 02/04/88

A Library of files and programmes to implement an 80 track DS DD drive

as drive B: on Amstrad CPC computers running CPM+

Note that no responsibility is accepted for any loss or damage occuring

from the use of these programmes or the advice contained herein !

These programmes and patches have been tested on a 6128 and work with no

known faults.

SAFETY:   IF YOU HAVE TO CONSTRUCT A POWER SUPPLY OR CONNECT ONE TO YOUR

AMSTRAD OR DISK DRIVE THEN BE CAREFUL !

Mains voltages which are present on the input side of power supplies

and in the monitor unit of the Computer are NOT FUNNY if you get

connected to them !

REMEMBER if in doubt DON'T !

CONTENTS:

BPAT.SUB        Submit file to auto patch C10CPM3.EMS

BPAT.HEX        data to be patched into above

BPAT.Z80        source code for data

BCPAT.SUB       Ditto for CPM3SEMI.EMS (as used by Dktronics

for their DRIVE C: Silicon disk)

BCPAT.HEX       ---- do ----

BCPAT.Z80       ---- do ----

MLOAD.COM       Public domain loader programme used by .SUB files

UNLOAD.COM      Public Domain un-loader programme ---- do ----

FM.COM          Formatter programme for use after patching EMS file

FM.Z80          Source code for above

BDISK.COM       Programme to patch XDPB for drive B: for 80 trk use

BDISK.Z80       Source code for above.

HOW TO GET STARTED WITH AN 80 TRACK DS DD DRIVE ON A 6128 ETC.

1.      Connect drive to Amstrad Disk expansion port using 34 way ribbon cable

and 34 way edge connectors. Note a Half-twist is required in the cable

as Amstrads pin no's do not coincide with Shugart's interface. So Pin

1 (Amstrad) is connected to Pin 34 (Shugart).

2.      Remove any pull-up resistor pack from the 80 trk drive as this will

load the Amstrads bus and is not needed as the internal drive is

'pulled up' already

3.      Check that your drive has a 'drive ready' signal on the appropriate

line to the Amstrad. If not one will have to be provided. This can

be achieved by a manual switch or better still a micro switch or

photodiode/phototransistor to detect the presence of a disk in the

drive or the drive door being closed.

4.      Make up a suitable power supply for the drive. It is said that the

Amstrad power supply can handle another drive but it is NOT a good

idea ! Better to make a power supply for the whole machine and forget

about the power from the monitor unit. The whole machine will work

better and quieter and will last longer. The power supply needs to

provide +5 volts and +12 volts well smoothed and regulated. Try

computer junk shops for switched mode power supplies as these are

particularly good and are often avaiable from people who have up-

graded IBM PC's with higher power supplies for hard disks etc.

5.      Having got all this together then plug it all in and turn on.

Try running DISCKIT3 to format the drive as a 40 track unit and

copy a few files back and forth to ensure the mechanics are working

ok. At this stgae you will only be accessing 40 tracks SS and will

show only 169k free or 178k free depending on whether you chose

system or data format in DISCKIT3.

6.      If all has gone well then copy all the files from this Library

onto a SYSTEM format disk along with your EMS file and SUBMIT.COM

7.      Boot up CPM+ from this disk and type:

SUBMIT BPAT<return> (SUBMIT BCPAT if you use Dktronics silicon disk)

The submit file should now operate and patch the EMS file for use

with the new B: drive When this operation is finished copy the new

EMS file onto a fresh system disk and re-boot CPM+

8.      If all boots up OK then put a disc in drive B: and type

FM<return)

You will now enter the formatting programme and should format

any disks to be used in the B: Drive at this time.

9.      When formatting is finished re-boot cpm+ again.

now type

BDISK<return>

You will be offered a menu of three formats under which the disk

can be used. Select one by pressing A,B or C

When the format has been patched into the XDPB the screen border will

change colour to indicate the High capacity format 80 track system

has been implemented.

10.     Check the free disk space on B: using SHOW or NSWP etc and verify

that it conforms to your selected choice under BDISK

11.     Try copying some files from A: to B: and back and check that they

do not become corrupted. If all is well then edit your PROFILE.SUB

start up file to include the line:

BDISK B (or A or C) so as to auto select the correct format for

using the B Drive.

12.     Enjoy using the disk space ! Up to 796 k free depending on choice

of format.

Good luck with the programmes.

.end#es to implement an 80 track DS DD drive

as drive B: on Amstrad CP