DIGITAL LOGIC SIMULATOR

The object of this program is the analysis of combinational

logic by generating a truth table for the logic equation that is

requested from the operator.  It is quicker and less error-prone

than doing it analytically.

To enter the equation with variables into an already

structured program, it was necessary to modify the program with

each equation entry.  The program is stopped, the equation is

entered at a specified line number, and then the program is

continued from that point using a RUN XXX.  On the next run, the

equation is entered at the same line number, wiping out the first

equation.

In writing the equation care must be taken to use the

correct expressions (GIGO).  Parentheses must be used to define

functions of NOT logic.  It is also advisable to use parentheses

to assist in defining logic groups.  When you have several logic

functions in one equation, it makes no difference whether you

start at the output and work towards the input, or start at the

input and work towards the output.  Usually the logic can be

broken into segements with separate equations, if more than six

inputs are required.

SAMPLE RUNS

What are the number of inputs ?2

Put in the equation by typing

440 X = < logic equation > <cr>

then type

RUN 420

OK

440 X = NOT(NOT(A OR B) OR (A AND B))

RUN 420

A	B	X

0	0	0

0	1	1

1	0	1

1	1	0

Is that all? NO

What are the number of inputs ?4

Put in the equation by typing

840 X = < logic equation > <cr>

then type

RUN 820

OK

840 X = NOT(NOT(NOT(A AND B) AND C) OR D)

RUN 820

A	B	C	D	X

0	0	0	0	0

0	0	0	1	0

0	0	1	0	1

0	0	1	1	0

0	1	0	0	0

0	1	0	1	0

0	1	1	0	1

0	1	1	1	0

1	0	0	0	0

1	0	0	1	0

1	0	1	0	1

1	0	1	1	0

1	1	0	0	0

1	1	0	1	0

1	1	1	0	0

1	1	1	1	0

Is that all? YES

Thank You

OK

#entheses

to assist in defining logic groups.  When you have several logic