If  you have a KAYPRO II and often wish that you had more room on

your disks or if you have said why didn't I wait and get a KP  4,

then the upgrade to a KAYPRO 8 is for you.  This upgrade is NOT a

massive  "rebuild/redesign"  your computer type of operation  nor

will you have to take out a home improvement loan to change  your

trusty old II into a 8.

The  purpose  of this modification is to upgrade the single  side

double density disk drives (Tandon 100-1) of the Kaypro II, which

have  a  capacity of 191K to double sided quad density drives  of

784K each.  The number of drives to be upgraded is up to you, one

original  +  one  800K,  two 800K,  four 800K,  two  800K  +  one

original,  two 800K + two 390K,  or just about any combination of

four  that you can think of.  In my system I have chosen the  two

800K disk drives.

Now how much is this going to upset the local economy;  less than

$500, check with your dealer on how much he will allow you for an

old II as a trade in on a 2X or a 4 (ouch!!!) but remember he has

to make a profit to stay in business.

WHAT YOU WILL NEED (up grade to two 800K drives)

ITEM                SOURCE              COST

Pro-Monitor 8       Micro Cornucopia    $49.95

DSQD disk drives    Back pages BYTE     $150 each (or less)

IC  74S04           Electronic parts    $1.00 (or less)

store or mail order

40 pin IC socket (1)     "              $1.00 (or less)

24 pin IC socket (1)     "                   "

20 pin IC socket (1)     "                   "

16 pin IC socket (1)     "                   "

14 pin IC socket (1)     "                   "

30 gauge wire

As  you can see the upgrade can be performed for less than  $355;

however there are certain risks (not to equipment) to your mental

health:  most  mail order houses have a minimum order of a  least

$10.00 (I was not able to locate the 74SO4 IC in any of the local

(El Paso) parts stores;  next the IC sockets must be of the  very

best quality, I used them for adapters (had some from Radio Shack

that were not satisfactory), the pins must have at least the same

length  and  thickness  as the legs of an IC chip  (not  easy  to

find).

The  Micro-C articles on performing this upgrade (issues 12,  15,

18  and  21) direct you to cut traces on the CPU  board  and  run

jumper wires to perform the modifications,  to me the finality of

cutting  a  trace is not pleasant.  What I have done  is  use  IC

sockets  with  selected  pins bent out and run the  jumper  wires

between the "bent out" pins, makes it easy to "un-modify"

PROCEDURE

1.    MONITOR ROM

A   Bend out pin 21 of a 24 pin socket (will be installed in

U47)  connect  to  pin 2 of a 20  pin  socket  (will  be

installed  in  U59),  do  NOT bend out pin 2 of  24  pin

socket.

B   Remove U47 and U59.

C   Install two modified sockets in  designated slots (U47 &

U59).

D   Install  new  U47  (Micro-C's  Pro-8),   reinstall   U59

(74LS373).

2.     BANK DECODE

A   Bend  out pin 1 of a 16 pin socket,  connect to pin 8 of

same 16 pin socket (not bent out).

B   Remove U60.

C   Install  U60  (74LS138) in  modified  socket,  reinstall

assembly in U60's slot.

3.     FLOPPY CONTROLLER

A   Bend out pin 13 of a 40 pin socket (will be installed in

U72),  bend out pins 5 and 6 of a 14 pin socket (will be

installed in U73.

B   Connect  pin  13  of 40 pin socket to pin 5  of  14  pin

socket (both pins bent out).

C   Connect  two  inch  length of wire to pin 6  of  14  pin

socket (bent out).

D   Remove U72 and U73.

E   Install modified sockets in designated slots.

F   Reinstall U72, install new 74S04 in U73.

G   Remove "ribbon cable" going to disk drives.

H   Connect  wire  from  U73 pin 6 to pin 32  of  J6  (cable

connection), by making a very small eyelet this wire can

be  connected  from  the top of the board and  will  not

interfere   with   the   cable   connection   when   its

reinstalled, later on.

There  is  a  much easier way to do all  this,  buy  a  Kit  from

Highland Microkit; P.O. Box 21; Highland, MD 20777. Dan Carl made

up  a kit for me which contained ALL the  sockets,   wiring,  the

74S04  (that hard to find rascal) and he also included a new  and

faster  Z80 PIO (U72).  Dan's sockets are the gold plate machined

pins, top quality.

You will still need the Micro-C Pro-8 ROM though.

I  installed  the do it yourself version first and  then  put  in

Highland's  kit,  I  was just not satisfied with the  quality  of

parts  that I could find on the local market.  Dan's kits are the

finest  that I have ever seen,  see Frank Fallon's review in  the

Nov-Dec  KUGRAM or KPII4M.DOC on DL4 for a review of his 4-5  MHz

speed up kit.

DISK DRIVES

There are several makes of drives on the market which will do the

job, see the back pages of BYTE and Computer Shopper. By a little

shopping you should be able to find some drives for $150 or less.

What  you  are  looking for is 96 TPI (tracks  per  inch)  double

sided.  My choice was the TEAC 55F's.  TEAC mounts their  circuit

boards  upside-down  from Tandon so keep this in mind if you  are

planning  to keep one of the old Tandons,  you will need  it  for

Uniform or MF disk conversions programs or what I may do later is

put a TEAC 55B in as the third drive. If you mix TEAC's and other

manufactures  the  ribbon  cable will require a  twist  that  may

resemble  a  "two and a half gainer with a  forward  twist".  The

terminator must be installed on the drive on the end of the daisy

chain,  makes no difference if its called A, B, C, or D, just the

last physical drive.  My A  drive is on top and the last drive on

the cable, it has the terminator. This way the cable makes a nice

loop without any undue stress or weird twisting.  Should you wish

to interchange disks with someone with a factory machine you must

be aware of the methods used in writing data to a disk.  The head

on the disk drive is really three heads;

_,           READ/WRITE HEAD

/    _________

/----|_________|<---------------TUNNEL ERASE

WRITE AREA <|  : |_________<--DATA is not erased

\----|_________|<---------------TUNNEL ERASE

\_^

The  relative motion is for the read to be moving right to  left,

the data section of the head will write the data in a rather wide

track  (up  to .021 in),  then the two "tunnel erase  heads  will

erase  the outer edges of the data track leaving only a track  of

.012  in.  These are the figures for a factory machine (48  tpi).

Now when you switch to a quad drive the max width of the track is

.0105 in, which does not remove all the old data which was put on

by  a  factory machine.  This will not cause any  trouble IF  the

disk is used only on a quad drive BUT if this disk is put back in

a  standard machine it will see the new data,  written by the  96

tpi drive, and some of the old data, written by the 48 tpi drive.

This tends to confuse the computer to no end.

TRACK WIDTHS

48 TPI

/---------/\----------\

|<-------.021 in----->|

|<--.012 in-->|

| G |             | B |

| U |  DATA AREA  | A |

| A |             | N |

| R |E5E5E5E5E5E5E| D |

| D |5E5E5E5E5E5E5|***|

|***|  |G| D |B|  |***|

|***|  |U| A |A|  |***|

|***|  |A| T |N|  |***|

|R| A |D| \______This area of .00075 in. will NOT

|D|   | |        be erased when the disk is used

|*|E5E|*|        in a 96 tpi drive.

|<A>|        A= .0065 in

|<- B ->|      B= .0105 in

\_______/

96 TPI

The way to over come this problem,  if you are going to  exchange

disks  between  machines,  is to erase the old disk with  a  bulk

eraser  and then reformat  it in your machine.  The old drives in

another  machine will be able to read your narrow data track with

no problem without seeing the "left overs". The potential problem

is when you write on a disk formatted by 48 tpi machine and  then

expect the disk to be read in a 48 tpi machine. The other machine

NOT  yours.   I'll  say some more about other makes of  computers

later on.

FLOPPY DISKS for use in a KAYPRO 8

I had a large number of single sided double density disks on hand

when  I made the modification to my machine (3+ months  ago)  and

simply started to use what was on hand. The predominant brand was

BASF  however I also hand many other brands on hand,  every  disk

that  I  have used in the new 800K system has  formatted  without

error.  A  few required two tries but only a few ( 2 or 3 out  of

100  ).  I've been using a BASF as my modem diary,  saving a temp

log for the terminal session and then editing out unwanted  stuff

after I get off the line ( MEX-SET ) no failures & the DIR tracks

were  written to and erased 5 to 6 times a week.  The system disk

is  a ValuLife,  same results.  Based on my experiences  I'd  say

continue  to  use what ever disk you are comfortable with on  the

factory drives.

WHAT DID NOT GO RIGHT

Not very much for this para: Plu*Perfect's enchantment to Perfect

Writer,  for some reason unknown to me,  will not work.  Kaypro's

MFDISK  is  the  only  other  item that  I've  found  not  to  be

compatible  with this mod.  MFDISK is hardware dependent and  the

hardware  was  changed --- therefore!  I do not have  a  copy  of

UNIFORM  but  MicroC  says  it will not run because  it  also  is

hardware dependent.

REFERENCES

Micro Cornucopia

issue # 12 page 24 "Kaypro column"

issue # 15 page 14 "Great Eight Kaypro"

issue # 17 page 50 "response to letter from Charles Dugan"

issue # 18 page  3 "Letter by Greg DeHoogh"

24 "Kaypro Column"

TEAC

FD 55 maintenance manual page 108

AND LAST

READ  READ  READ every thing that comes from MicroC and the drive

manufacture.  This  is an easy mod to do IF and only if  you  can

follow directions, no short cuts permitted.   I like it.

If you are thinking about installing one of the CPU speedup kits

on the market, you should consider the kit sold by HIGHLAND

MICROKIT  P.O. BOX 21 HIGHLAND, MD 20777. I installed of these

kits in late June and its worked perfect since then. The nice

thing about these kits is that NO soldering or cutting of PWB

traces is required on an older KP II. This kit is the same as the

speedup modification described in MICRO C #12 (Jun 83). The only

program that has not worked at 4 Mz is PLU*PERFECT's fast copy,

at 56 sec to copy a disk who cares! My KP II will run for about

one hr at 5 Mz but then it craps out but at 4 Mz it'll go all

#t 56 sec to copy a disk who cares! My KP II will run for about

one hr at 5 Mz but then it craps out but at 4 Mz it'l