**>> ZX.DOC <<**

ZX.COM is a horiZontal directory program, and also performs

elementary file management operations.  File names are listed in

horizontal alphabetical order.  Here's what the screen looks like:

A>zx D:

> CHAR    .DEF  1k > CHAR    .GRK  1k > CHAR    .GTH  1k > CHAR    .ZIG  1k

> FIND51  .COM  3k > FINDBD54.COM  2k > INITDISK.COM  5k > MERGPRIN.OVR  8k

> NSWP    .COM 12k > PRN     .COM 29k > RAMDISK .COM  9k > SETDISK .COM  6k

> TOON12  .COM  1k > UNERA   .COM  1k > USFT1C  .COM  6k > WS      .COM 14k

> WSJ     .COM 14k > WSMSGS  .OVR 26k > WSOVLY1 .OVR 27k > ZX      .COM  1k

D: 254k total/170k used >> 84k left

A>

ZX also contains a file erase function, a type function for

viewing files, and a rename function for changing file names.

ZX.COM is smaller than 1k, the minimum CP/M file size.  ZX's

small disk footprint is well-suited to any situation where space

should be conserved, and its horizontal output is well suited for

small screens (such as LCD's).  However, ZX should work on any

CP/M 2.x system which has a Z80 CPU.

**>> USING ZX <<**

DIRECTORY functions are performed in standard CP/M style:

A>ZX                                    -> directory of A:

A>ZX D:                                 -> directory of D:

A>ZX a.fl                               -> show A.FL only

A>ZX *.COM                              -> all .COM files on A:

A>ZX B:??.*                             -> B:files with 1 & 2

letter file names

ERASE is indicated by putting e or E after the file name:

A>ZX bad.txt e                          -> erases A:BAD.TXT,

then shows directory

A>ZX B:*.bak e                          -> erases all .BAK

files on B:, confirms

with B: directory

TYPE is initiated by t or T after the file, and stopped by ^X:

A>ZX bad.txt T                          -> each keypress

types one sector of

BAD.TXT, ^X exits

to a directory

RENAME requires two file names on the command line, then r or R:

A>ZX apt.txt bad.txt r                  -> renames APT.TXT to

BAD.TXT, then shows the

directory to confirm

ZX.COM's operations are all duplicated in CP/M's standard CCP,

which has DIR, ERA, TYPE, and REN.  Why use ZX?

The reason lies in the better information supplied in ZX's

directory, and in the integration of the other functions with a

directory display.  I always check the directory after erasing a

file, and ZX's automatic confirmation of an erase or rename

operation saves keystrokes many times a day.

**>> ERROR MESSAGES <<**

=No file=  indicates that a filename supplied in a rename, erase,

or type operation does not exist.  The same message appears when

a directory is requested for an empty disk.  A =No file= is

always followed by a directory display in order to assist the

user in typing the correct file name.

=File exists=  indicates that you have tried to rename a file to

a file name which already exists.  The message is (of course)

followed by a directory.

=No */?=  is the most mysterious of the error messages, but means

that an ambiguous file name (containing * or ?) has been supplied

where an unambiguous file name must be.  For example, ambiguous

file names should never be supplied for renaming, because the

directory will be scrambled.  ZX will trap most such attempts,

without restricting the uses of amgiguity in directory and erase

operations.  To assist in finding the specific file name, =No */?=

is followed by a full directory (surprise!).

**>> APPLICATION NOTES  <<**

ZX uses many tricks to stay small.  It uses Z80 rather than 8080

operations where they save disk space, and thus will run only on

the Z80.  Fortunately, this covers most modern systems.  It

builds its directory table above itself, so that it does not have

to reserve disk space for it.  This means that ZX will overwrite

the kbyte or so above its last load address during operation.

Even ZX's internal stack is exported to the unused interrupt area

of the zero page, so as to require no reserved disk space.

ZX is an inheritor of the idea expressed in DO.COM, which also

combines several utilities into minimal disk space.  There is a

subtle benefit of this idea which one only appreciates with use.

One kbyte programs can seem to load and execute instantaneously,

which is definitely the way a computer should behave.  I hope you

find ZX useful.

August, 1985                                 Mike Yarus

2231 16th Street

Boulder, CO 80302

##################################################################################################