DSKON1, DSKON2, DSKOFF1, and DSKOFF2 are utilities that will

shut down the disk drives in perfect writer. The reason that

there are two sets of files, is that there are two bios's on the

Kaypro II. The second bios comes in when the configure program is

run. The configure program actually writes a new bios to the

disk. The utilities load themselves just below the  CCP.

Steve Haga

[Librarian's note:  Steve obtained these programs from someone in

a users' group he is helping start elsewhere.

Use the one that works for you by invoking it under CP/M

before entering PW.  Drive motors will then stop spinning after

swaps, etc. rather than menacing your sanity by continuing.

Disassembly reveals they work by setting up a partial BIOS

replacement under the CCP which is called by all CONOUT BIOS

calls.  The Drive off bit on the system control port is strobed

every time a character is sent to the console.  Normally, the

drive is strobed on CONIN calls, which PW does only after a key

is struck (see the CBIOS on an earlier user's group disk).  Long

run effect of this method of shutting the drive off is unclear.

Use only at your own risk!

Steve McMahon]

#comes in when the configure prog