INSTALLING THE NEW BIOS ON YOUR KAYPRO II

(AND CHANGING THE COLD BOOT BAUD RATE ON THE NEW VERSION)

WHAT'S NEW ABOUT THE NEW BIOS?

(AND WHAT'S A `BIOS' ANYWAY?)

The latest version of CP/M on the KAYPRO II is different

in three significant ways from the CP/M issued on earlier

KAYPRO's:

(1) The drives will 'time out' and stop spinning in many

applications programs such as Perfect Writer where previously

they would just keep on spinning until a character was struck on

the keyboard;

(2) An autoload on cold start feature is enabled.  This allows

you to specify a command line to be executed on a cold start

(when the machine is first turned on or the reset button is

pushed).  Significantly, this command line will only be executed

on a cold start.  Many autoload techniques execute their autoload

command every time they warm boot -- this can make them annoying

or worthless.  The autoload feature works in conjunction with the

new COPY program, which allows the setting of the command line to

be executed;

(3) The machine may be set up to come to life with a serial baud

rate of something other than 300 Baud -- a real convenience if

you have a fast printer or modem running off your serial port.

The technique for making the change, though, is less than

convenient (it will be reviewed below);

All of these features are results of small changes in the

Basic Input Output System (BIOS) portion of the system.  The BIOS

is the portion of CP/M written by the manufacturer for the

particular machine to tell the rest of CP/M how to use the

particular hardware.  So, none of the new features are changes in

the portions of CP/M provided by Digital Research:  rather they

are improvements in the way NLS has implemented CP/M on the

KAYPRO.

What this means is that you don't have to go out and by a

new copyrighted and serialized version of CP/M to get the upgrade

features.  Instead, you can simply replace the BIOS in your

current version of CP/M with an updated BIOS and get all of the

new features.  A step-by-step procedure for making this

replacement -- which can be followed by anyone who has understood

this discussion so far -- is detailed below.

You should treat the result of this procedure as a new,

master version of CP/M.  It may be moved on to other disks with

SYSGEN, just as with the earlier version of CP/M.

A PLUG FOR ZCPR

As long as your messing with your copy of the system

anyway, you might as well install ZCPR in the process.  ZCPR (Z80

Command Processor Replacement) is a public domain replacement for

Digital Research's Console Command Processor (CCP).  The CCP is

the section of the system that you actually interact with when

you use CP/M: DIR, ERA, TYPE, REN and SAVE all reside within CCP

as does the code which fetches any transient program you wish to

run and places it in memory.  The features of ZCPR are

extensively discussed in its BAKUP Volume I document and have

been reviewed in the newsletter more than once.  Look up these if

you are interested.  ZCPR is highly recommended (but, as with all

BAKUP's software, completely unwaranteed!).  The installation

technique below should be virtually painless.

NEW BIOS INSTALLATION INSTRUCTIONS

This procedure will require 2 disks.  On one of the disks

you should have a copy of the system, NEWBIOS.COM, ZCPR.COM

(optional), DDT.COM (if you wish to alter the cold boot baud

rate) and SYSGEN.COM.  The other disk should either be a blank or

should contain nothing important in case anything goes wrong (it

must, though, be formatted).  Put the drive with the system and

NEWBIOS.COM in the A drive and the blank in B.  The disk in B

will be receiving the new version of CP/M.

Now, follow the transcript below.  Curly brackets `{}'

indicate you should enter what's between them.  `CR' means enter

a return.  Square brackets `[]' mark off my comments and should

not be typed in.

----------------------------------------------------------------------

[TRANSCRIPT OF NEW BIOS INSTALLATION]

[First, pull a copy of the system into memory from A:]

A>{sysgenCR}

KAYPRO SYSGEN VER 2.2

SOURCE DRIVE NAME (OR RETURN TO SKIP){aCR}

SOURCE ON A, THEN TYPE RETURN{CR}

FUNCTION COMPLETE

DESTINATION DRIVE NAME (OR RETURN TO REBOOT){CR}

Warm Boot

[An image of the system now sits in the main part of RAM awaiting

our changes.  NEWBIOS.COM pulls the new copy of the BIOS into

RAM, then moves it into place on to of the image of the old BIOS.]

A>{newbiosCR}

New BIOS installed in sysgen image.

[-----------------Optional--------------------------------------------]

A>{zcprCR}         [For those wishing to install ZCPR]

[---------------------------------------------------------------------]

[-----------------Optional--------------------------------------------]

A>{save 34 newcpm.comCR}   [For those wishing to change the cold boot baud]

[---------------------------------------------------------------------]

[The sysgen image of the system has been altered and now we'll

write it to the B: drive disk.]

A>{sysgenCR}

KAYPRO SYSGEN VER 2.2

SOURCE DRIVE NAME (OR RETURN TO SKIP){CR}

DESTINATION DRIVE NAME (OR RETURN TO REBOOT){bCR}

DESTINATION ON B, THEN TYPE RETURN{CR}

FUNCTION COMPLETE

DESTINATION DRIVE NAME (OR RETURN TO REBOOT){CR}

Warm Boot

[All done unless you wish to change the default warm boot serial

baud rate to something other than 300 baud.]

[-------------------------------------------------------------------]

[           CHANGING THE DEFAULT COLD BOOT BAUD

REQUIRES A DISK FILE OF THE SYSGEN SYSTEM IMAGE OF THE NEW

SYSTEM (SUCH AS THE NEWCPM.COM FILE GENERATED ABOVE).]

[First, load the sysgen image into RAM for alteration.]

A>{ddt newcpm.comCR}

DDT VERS 2.2

NEXT  PC

2300 0100

[Now, take a look at the image of the BIOS.]

-{d1f80CR}

1F80 C3 48 FA C3 99 FA C3 F3 FA C3 09 FB C3 2F FB C3 .H.........../..

1F90 48 FB C3 43 FB C3 3E FB C3 7C FB C3 80 FB C3 84 H..C..>..|......

1FA0 FB C3 88 FB C3 8C FB C3 90 FB C3 98 FB C3 65 FB ..............e.

1FB0 C3 A8 FB 81 00 0B 0A 08 0C 30 31 32 33 34 35 36 .........0123456

1FC0 37 38 39 2D 2C 0D 2E 05 CD 78 FB AF 32 04 00 3A 789-,....x..2..:

1FD0 33 FA 32 03 00 3A 47 FA D3 00 CD CF FB 1A 0D 0A 3.2..:G.........

1FE0 4B 41 59 50 52 4F 20 49 49 20 36 34 6B 20 43 50 KAYPRO II 64k CP

1FF0 2F 4D 20 76 65 72 73 20 32 2E 32 0D 0A 00 3E C3 /M vers 2.2...>.

2000 21 03 FA 32 00 00 22 01 00 21 06 EC 32 05 00 22 !..2.."..!..2.."

2010 06 00 3A 04 00 4F C3 00 E4 CD 78 FB CD CF FB 0D ..:..O....x.....

2020 0A 57 61 72 6D 20 42 6F 6F 74 0D 0A 00 31 00 01 .Warm Boot...1..

2030 0E 00 CD 80 FB 01 00 00 CD 84 FB 21 00 E4 22 14 ...........!..".

[Byte 1FC7H should be a `05' for 300 Baud. If it isn't,

something's wrong -- cancel the whole mess with a ^C.]

[Now, we'll change that byte to reflect the speed we want the

serial port to run at:

Baud:		Byte:

300		5

600		6

1200		7

2400		A

4800		C

9600		E

19.2k		F

In the example below, I changed the port to run at 1200 baud by

entering a 7, a return, a period, and a return.]

-{s1fc7CR}

1FC7 05 {7CR}

1FC8 CD {.CR}

-{g0CR}		[exit DDT.COM]

Warm Boot

[Now, install the new CP/M on b:]

A>{sysgenCR}

KAYPRO SYSGEN VER 2.2

SOURCE DRIVE NAME (OR RETURN TO SKIP){CR}

DESTINATION DRIVE NAME (OR RETURN TO REBOOT){bCR}

DESTINATION ON B, THEN TYPE RETURN{CR}

FUNCTION COMPLETE

DESTINATION DRIVE NAME (OR RETURN TO REBOOT){CR}

Warm Boot

A>

[ALL DONE]

------------------- END TRANSCRIPT ------------------------

*********

REMEMBER TO TEST THE NEW VERSION OF THE SYSTEM BEFORE

YOU TRUST IT WITH VALUABLE DATA!!!!!!!!

*********

##################################################################################################################