***************************************************************

OPERATORS MANUAL FOR THE MICROCOSM ASSOCIATES

MEMORY DIAGNOSTIC, VERSION 1.0

***************************************************************

COPYRIGHT 1979,1980 BY MICROCOSM ASSOCIATES

DONATED TO THE "SIG/M" CP/M USER'S GROUP, BY KELLY SMITH

CP/M IS A REGISTERED TRADEMARK OF DIGITAL RESEARCH

---------------------------------------------------------------

BAY AREA KAYPRO USERS & PROGRAMMERS

KAYPRO II IMPLEMENTATION NOTES

To run this diagnostic on the Kaypro, you need have a

printer or console you can connect to either the parallel or

serial ports to capture the program output.  This is necessitated

by the way the KP handles the CRT (memory mapped output using the

PROM).  Since it would have been difficult to include all the

code necessary to put program output on the screen (normal BIOS

functions are unavailable when that section of memory may be

among those tested), I took the easy way out and assumed another

device was available.

If the port you are using is the serial, you should use

MDIAGTTY.COM, and if it's the parallel, use MDIAGLPT.COM.

In any case, the program wants its input from the KP

keyboard.

In the automatic mode, the program runs extensive tests

on the middle 3/4 of memory (1000H to C000H, which is just about

all the TPA).  This takes quite a while to run.  If you wish to

check beyond C000H or to run a faster test, use the non-auto

mode.  In doing so, note that the top two bytes of RAM (FFFEH and

FFFFH) seem unaccessible for some of the tests--the program loops

if asked to test them.

Steve McMahon

Berkeley

March, 1983

---------------------------------------------------------------

1.0

DIAGNOSTIC TEST "START-UP" EXECUTION

1.1

THE  TEST  WILL INITIALLY "SIGN ON",WITH THE FOLLOWING  MESSAGE

DISPLAYED ON THE REMOTE CONSOLE:

MICROCOSM ASSOCIATES MEMORY DIAGNOSTIC VER.1.0 (C) 1979

TOP OF MEMORY=NNNN

MEMORY QUALIFICATION TEST IS IN PROGRESS

ANY ERRORS OCCURING DURING THE "MEMORY QUALIFICATION TEST" WILL

BE DISPLAYED AS FOLLOWS:

ERROR AT ADDRESS=NNNN  PATTERN STORED=NN  PATTERN READ=NN

BIT(S) IN ERROR=N,N,N,N,N,N,N,N

THE  DIAGNOSTIC  PROGRAM  WILL  NOW  PROCEED  TO  DISPLAY   THE

FOLLOWING MESSAGE ON THE REMOTE SYSTEM CONSOLE:

MEMORY TEST IN AUTO MODE (Y OR N)?

(REFERENCE SECTION 2.1 C FOR ADDITIONAL INFORMATION)

2.0

DIAGNOSTIC COMMAND MONITOR

2.1

WHEN  THE  "-"  PROMPT CHARACTER IS  DISPLAYED,  THE  FOLLOWING

KEYBOARD ENTRIES MAY BE MADE FOR TEST/DIAGNOSTIC SELECTIONS.

A.  "G" CAUSES "GO ADDRESS=",  ALLOWING EXECUTION OF CODE AT ANY

ADDRESS IN ROM/RAM MEMORY.

B. "M" CAUSES "MEMORY ADDRESS ", TO ALLOW DISPLAYING ANY ADDRESS

IN ROM/RAM MEMORY. FOR EXAMPLE:

-MEMORY ADDRESS=NNNN (CR)

NNNN=NN    (SP)

NNN1=NN AA (WHERE "AA" IS AN OPERATOR ENTRY)

NNN2=NN  ^ (WHERE  "^" IS AN  OPERATOR  ENTRY  TO  EXAMINE

"PREVIOUS")

NNN1=AA    (SP)

NNN2=BB E  (WHERE "E" INDICATES "MEMORY ERROR" ON ENTRY)

NNN3=NN    (CR)

-            (WHERE  "-"  IS  A  RETURN  TO  THE  MONITOR  BY

CARRIAGE-RETURN)

C.  "T"  CAUSES  "TEST  MEMORY IN AUTO MODE (Y  OR  N)?"  TO  BE

DISPLAYED  ON  THE REMOTE CONSOLE.ENTERING "Y" ON THE  REMOTE

CONSOLE KEYBOARD WILL CAUSE THE FOLLOWING DISPLAY TO APPEAR:

NO MEMORY BLOCKS DROPPED

TESTING MEMORY FROM 1000 TO 2000

PASS=0000  TOTAL ERRORS=0000

RANDOM PATTERNS TEST

THE  TEST  WILL PROCEED TO TEST WITH "WRITE  SATURATION  TEST",

"GALLOPING  PATTERNS TEST","STATIC CHECK  CYCLE","CHECKING  DATA

RETENTION","GALLOPING  COLUMNS  TEST",   AND  "WALKING  PATTERNS

TEST".  THE  TEST  WILL THEN PROCEED TO THE NEXT 4K BYTE  MEMORY

BLOCK.

A  "N"  RESPONSE TO "TEST MEMORY IN AUTO MODE (Y OR  N)?"  WILL

CAUSE THE FOLLOWING DISPLAY TO APPEAR ON THE REMOTE CONSOLE:

TEST "ALL","SELECT",OR "MONITOR" (A,S OR M)?

ENTRY  OF  "S" ON THE KEYBOARD ALLOWS  "SELECTION"  OF  TESTING

MEMORY  FROM  ADDRESS  1000 HEX TO FFFF HEX,WITH  THE  FOLLOWING

DISPLAY  APPEARING  ON THE REMOTE  CONSOLE:  (NOTE:   A  MINIMUM

ADDRESS DIFFERENTIAL OF THREE ADDRESS'S MUST BE UTILIZED BY  THE

OPERATOR,  AND MEMORY MAY NOT BE TESTED BELOW RAM MEMORY ADDRESS

1000 HEX.

LOW MEMORY ADDRESS=NNNN (CR)

HIGH MEMORY ADDRESS=NNNN (CR)

TEST OPTIONS ARE:

01-GALLOPING PATTERNS TEST

02-GALLOPING COLUMNS TEST

03-WALKING PATTERNS TEST

04-RANDOM PATTERNS TEST

05-WRITE STAURATION TEST

06-STATIC CHECK CYCLE TEST

ENTER TEST NUMBER(01,02,03,04,05 OR 06)=NN

ENTER PATTERN (00-FF)=NN TEST IS IN PROGRESS

PROGRAM TEST EXECUTION BEGINS AT THE TWO DIGIT ENTRY FOR "ENTER

PATTERN (00-FF)=" BY DISPLAYING "TEST IS IN PROGRESS".  ENTERING

"A" FOR "ALL",AUTOMATICALLY BEGINS EXECUTION BY DISPLAYING "TEST

OPTIONS  ARE:",ASSUMING THAT THE TEST ADDRESS RANGE IS FROM 8000

HEX TO BFFF HEX.

ENTERING  "M"  FOR  "MONITOR",CAUSES  EXITING  TO  THE  COMMAND

MONITOR   (SEE   SECTION  1.1  FOR   DETAILED   EXPLANATION   OF

OPERATION),AND  WILL  PROMPT  WITH A "-" CHARACTER  TO  INDICATE

MONITOR MODE.

3.0

RAM MEMORY TEST ERROR MESSAGES

A. RANDOM PATTERNS MEMORY TEST

MEMORY ERRORS OCCURING DURING THE RANDOM PATTERNS TEST WILL  BE

DISPLAYED ON THE REMOTE CONSOLE AS FOLLOWS:

ERROR AT ADDRESS=NNNNN PATTERN STORED=NN  PATTERN READ=NN

BIT(S) IN ERROR=N,N,N,N,N,N,N,N

NOTE, THAT ON TRANSIENT/DYNAMIC TYPE ERRORS, THE PATTERN STORED

MAY  MATCH  THE PATTERN READ AND NO BITS WILL BE IN  ERROR.  THE

ERROR  DETECTION  LOGIC OF THE TEST SOFTWARE IS  SUCH  THAT  ANY

IMMEDIATE  MISMATCH IS DETECTED BUT THE ERROR CELL STATE IS  NOT

SAVED.  SUBSEQUENT SOFTWARE PROGRAM DELAYS LATER, THE ERROR CELL

ADDRESS  CONTENT  IS  AGAIN  EXAMINED FOR  THE  MISMATCH  TO  BE

DISPLAYED,  BUT  THE CELL IS NOW CORRECT.  THIS INDICATES THAT A

TRANSIENT/DYNAMIC  ERROR HAS OCCURRED.  THE TEST SEQUENCE IS  AS

FOLLOWS:

1.   RANDOM  DATA PATTERNS ARE WRITTEN THRUOUT THE 4K BYTE  CELL

MEMORY BLOCK.

2.  ALL 4K BYTE CELLS ARE READ.

3.  ALL 4K BYTE CELLS ARE COMPLEMENTED.

4.  ALL 4K BYTE CELLS ARE READ.

5.  ALL 4K BYTE CELLS ARE ALTERNATELY WRITTEN WITH 55 HEX AND AA

HEX.

6.  ALL 4K BYTE CELLS ARE READ.

7.  ALL 4K BYTE CELLS ARE COMPLEMENTED.

8.  ALL 4K BYTE CELLS ARE READ.

9.  ALL 4K BYTE CELLS ARE WRITTEN WITH ALL 1'S.

10.  ALL 4K BYTE CELLS ARE READ.

11.  ALL 4K BYTE CELLS ARE COMPLEMENTED.

12.  ALL 4K BYTE CELLS ARE READ.

B.    WRITE SATURATION TEST

MEMORY ERRORS OCCURING DURING THE WRITE SATURATION TEST WILL BE

DISPLAYED ON THE REMOTE CONSOLE AS FOLLOWS:

ERROR AT ADDRESS=NNNN  READ BACK NN ,EXPECTED NN

THE FAILURE TYPE REVEALED IS SLOW SENSE AMPLIFIER RECOVERY  DUE

TO SUSTAINED "CHARGING" OF 0'S OR 1'S ON THE SENSE AMPLIFIER

CIRCUITRY.

THE TEST SEQUENCE IS AS FOLLOWS:

1.  WRITE  A "BACKGROUND" PATTERN OF ALL 0'S IN A 4K BYTE  BLOCK

REPEATEDLY FOR 100 WRITE PASSES

2. WRITE ALL 1'S IN A 4K BYTE BLOCK FOR 1 WRITE PASS ONLY.

3. SEQUENTIALLY READ ALL BYTES IN THE 4K BYTE BLOCK.

4.  WRITE  A "BACKGROUND" PATTERN OF ALL 1'S IN A 4K BYTE  BLOCK

REPEATEDLY FOR 100 WRITE PASSES.

5. WRITE ALL 0'S IN A 4K BYTE BLOCK FOR 1 WRITE PASS ONLY.

6. SEQUENTIALLY READ ALL 4K BYTES IN THE 4K BYTE BLOCK.

C.    GALLOPING PATTERNS MEMORY TEST

MEMORY  ERRORS  OCCURING DURING THE GALLOPING  PATTERNS  MEMORY

WILL BE DISPLAYED ON THE REMOTE CONSOLE AS FOLLOWS:

ERROR READING OTHER CELL

TEST CELL=NNNN,  OTHER CELL=NNNN

PATTERN STORED=NN  PATTERN READ=NN

THE  "OTHER" CELL (WRITTEN FROM A BACKGROUND PATTERN OF 0'S  OR

1'S) HAS BEEN DISTRUBED BY WRITING INTO THE "TEST" CELL.

ALTERNATELY,THIS MESSAGE MAY APPEAR:

ERROR READING TEST CELL

TEST CELL=NNNN,  OTHER CELL=NNNN,

PATTERN STORED=NN  PATTERN READ =NN

THE  TEST CELL HAS BEEN DISTURBED BY WRITING INTO SOME  "OTHER"

CELL.   THE  FAILURE  TYPE  REVEALED  IN  EITHER  ERROR  MESSAGE

INDICATES EITHER UNSATISFACTORY ADDRESS TRANSITIONS BETWEEN EACH

CELL  AND  EVERY OTHER CELL,  SLOW SENSE AMPLIFIER  RECOVER,  OR

DESTRUCTION OF DATA DUE TO NOISE COUPLING BETWEEN CELLS WITHIN A

COLUMN. THE TEST SEQUENCE IS AS FOLLOWS:

1.  A "BACKGROUND" PATTERN OF ALL 1'S IS WRITTEN INTO A 256 BYTE

PORTION OF A 4K BYTE BLOCK OF MEMORY.

2.   THE  COMPLIMENT OF THE TEST CELL IS WRITTEN INTO  THE  TEST

CELL,  AND  ALTERNATELY THE TEST CELL IS READ AND EVERY OTHER

CELL.

3.  THE TEST CELL IS WRITTEN TO IT'S ORIGINAL STATE.

4.  THE NEXT SEQUENTIAL CELL IS TESTED AS IN STEP 2 AND 3.

5.   A "BACKGROUND" PATTERN OF ALL 0'S IS WRITTEN INTO THE  SAME

256 BYTE PORTION OF THE 4K BYTE BLOCK OF MEMORY.

6.  STEPS 2,3, AND 4 ARE REPEATED.

7.  STEPS 1 THRU 6 ARE REPEATED FOR THE NEXT SEQUENTIAL 256

BYTES IN THE 4K BYTE BLOCK.

D.  STATIC CHECK CYCLE.

UTILIZED IN THE MEMORY TEST AUTOMATIC MODE ONLY, A STATIC CHECK

CYCLE WILL OCCUR AS FOLLOWS:

ANY  FURTHER MEMORY READ OR WRITE OPERATION IS  TERMINATED  FOR

ONE  MINUTE.   AFTER THIS DELAY TIME,  THE LAST STATE OF  MEMORY

TESTING PATTERNS FROM THE GALLOPING PATTERNS MEMORY TEST IS READ

FOR  THE  4K  BYTE  BLOCK TO TEST FOR  POSSIBLE  ERRORS  DUE  TO

RETENTION  LOSSES.  THE  FOLLOWING  MESSAGE WILL APPEAR  ON  THE

REMOTE CONSOLE:

CHECKING DATA RETENTION

ANY  DATA  RETENTION  ERROR DETECTED WILL BE  DISPLAYED  ON  THE

REMOTE CONSOLE AS FOLLOWS:

DATA RETENTION ERROR AT ADDRESS=NNNN

EXPECTED "00" DATA, READ BACK NN

E. GALLOPING COLUMNS MEMORY TEST

ERROR  MESSAGES  ARE  IDENTICAL  TO  THOSE  DISPLAYED  FOR  THE

GALLOPING PATTERNS MEMORY TEST. THE TEST SEQUENCE IS AS FOLLOWS:

1.  A "BACKGROUND" PATTERN OF ALL 1'S IS WRITTEN INTO A 64 BYTE

BYTE PORTION OF A 4K BYTE BLOCK OF MEMORY.

2.  THE COMPLEMENT OF THE TEST CELL IS WRITTEN INTO THE TEST

CELL, FOR THE CURRENT TEST COLUMN.

3.   TEST  CELL ROW'S ARE READ BY ALTERNATELY READING  THE  TEST

CELL  AND  LOCATIONS +64 ADDRESS'S FROM THE TEST  CELL  (E.G.

TEST CELL +64 (N), WHERE N=1,2,3,.... ,63).

4.   READ  THE TEST CELL,  THEN WRITE THE TEST CELL'S COMPLEMENT

INTO  THE  TEST  CELL,   SO  THAT  IT  IS  IN  IT'S  ORIGINAL

"BACKGROUND" STATE.

5.  READ TWO CELLS FOLLOWING THE TEST CELL FOR REFRESH CHECK.

6.   REPEAT STEP 5 FOR ALL CELLS OF THE CURRENT TEST COLUMN, AND

EVERY   OTHER   COLUMN   (E.G.   READ   CELLS   3,6,9,....63;

2,4,9,...62; 1,4,7,.....61; ETC., ETC.)

7.  REPEAT STEPS 2 THRU 6 FOR ALL COLUMNS.

8.  WRITE A "BACKGROUND" PATTERN OF ALL 0'S FOR 64 BYTES IN THE

4K BYTE MEMORY BLOCK.

9.  REPEAT STEPS 2 THRU 7.

10.   REPEAT STEPS 1 THRU 9 FOR THE NEXT SEQUENTIAL 64 BYTES  IN

THE 4K BYTE MEMORY BLOCK.

ALL  FAILURE  TYPE  REVEALED  INDICATES  EITHER  UNSATISFACTORY

ADDRESS  TRANSITIONS BETWEEN EVERY CELL AND THE POSITION OF  THE

CELL'S  ROW,  DESTRUCTION  OF STORED DATA DUE TO NOISE  COUPLING

AMOUNG CELLS IN THE SAME COLUMN, OR REFRESH ERRORS.

F.  WALKING PATTERNS MEMORY TEST

MEMORY  ERRORS  OCCURRING DURING THE  WALKING  PATTERNS  MEMORY

TEST WILL BE DISPLAYED ON THE REMOTE CONSOLE AS FOLLOWS:

ERROR AT ADDRESS=NNNN READ BACK NN, EXPECTED NN

LAST ADDRESS WRITTEN WAS NNNN, PATTERN WAS NN

THE TEST SEQUENCE IS AS FOLLOWS:

1.   WRITE A "BACKGROUND" PATTERN OF ALL 1'S FOR 256 BYTES IN  A

4K BYTE MEMORY BLOCK.

2.  WRITE THE COMPLEMENT OF THE TEST CELL INTO THE TEST CELL.

3.  READ THE REMAINING 256 BYTES, IN THE 4K BYTES OF THE BLOCK.

4.  READ THE TEST CELL.

5.   WRITE THE COMPLEMENT OF THE TEST CELL BACK TO IT'S ORIGINAL

"BACKGROUND" STATE.

6.  REPEAT STEPS 2 THRU 5.

7.  WRITE A "BACKGROUND" PATTERN OF ALL 0'S FOR 256 BYTES IN THE

4K BYTE MEMORY BLOCK.

8.  REPEAT STEPS 2 THRU 6.

9.   REPEAT  STEPS 1 THRU 8 FOR THE NEXT SEQUENTIAL 256 BYTES IN

THE 4K BYTE MEMORY BLOCK.

THE  FAILURE  TYPES  REVEALED  ARE  INTERNAL  MULTIPLE  ADDRESS

SELECTION,DESTRUCTION  OF  STORED  DATA DUE  TO  NOISE  COUPLING

WITHIN A COLUMN,AND SLOW SENSE AMPLIFIER RECOVERY.

G.  OPERATOR ERROR MESSAGES

ANY NON-HEXIDECIMAL KEYBOARD ENTRIES ON THE REMOTE CONSOLE WILL

BE  PROMPTED WITH A "?" CHARACTER,  AND A RETURN TO THE  COMMAND

MONITOR.   ANY  ATTEMPT  TO  TEST  "OUT  OF  RANGE"  OF  ADDRESS

SELECTIONS WILL PROMPT WITH:

INVALID MEMORY ADDRESS

TESTING  OF MEMORY IS NOT ALLOWED BELOW ADDRESS 1000,  OR  ABOVE

FFFF,  ALSO  THE LOW MEMORY ADDRESS MAY NOT BE GREATER THAN  THE

HIGH MEMORY ADDRESS, ANY ADDRESS RANGE FROM 1000 HEX TO FFFF HEX

MAY BE TESTED .HOWEVER,  A MINIMUM DIFFERENCE IN ADDRESS OF 3 IS

REQUIRED  DUE  TO THE "PUSH/POP" METHOD OF MEMORY  TEST  BY  THE

RANDOM  PATTERNS  MEMORY TEST ("PUSH AND POP"  INSTRUCTIONS  ARE

UTILIZED  FOR  MEMORY  ACCESS'S BECAUSE THESE  TWO  INSTRUCTIONS

PERFORM THE FASTEST ACCESS OF MEMORY).

# MEMORY BLOCK.

THE  FAILURE  TYPES  REVEALED  ARE  INTERNAL  MULTIPLE  ADDRESS

SELE