DOCUMENTATION FOR MEMRS.COM

Introduction

------------

MEMRS   is  an  exhaustive  memory  test  program  for  CP/M

environments.  It  tests from the top of itself to the bottom  of

the CP/M BIOS or alternatively, the BDOS.

Usage

-----

MEMRS  [-R][-B]

Where

[-R]           is  an optional "Random  number  only"  test

specifier

[-B]           is an optional "Test to BDOS" specifier

Examples

--------

MEMRS

This will invoke the program with no special options.

MEMRS -R

This will invoke the program for the random number test

only.

MEMRS -B

This will invoke the program for default testing to the

bottom of the BDOS and use the BDOS for I/O rather than

default test to the bottom of the BIOS and use the BIOS

for I/O.

MEMRS -BR  or  MEMRS -RB  or  MEMRS -B -R  or  MEMRS -R -B

This will invoke the program for default testing to the

bottom  of  the  BDOS as above AND random  number  only

test.

Error Messages

--------------

If  any memory location fails any of the component parts  of

the memory test an error message will be displayed as follows:-

LOCATION  SHOULD BE           WAS

6000    00  0000 0000     FF  1111 1111

If there are multiple bad memory locations up to 18 will  be

displayed  before the display halts and prompts for a keypress to

display further errors.

LOCATION  SHOULD BE           WAS

7000    00  0000 0000     FF  1111 1111

7001    00  0000 0000     FF  1111 1111

7002    00  0000 0000     FF  1111 1111

7003    00  0000 0000     FF  1111 1111

7004    00  0000 0000     FF  1111 1111

7005    00  0000 0000     FF  1111 1111

7006    00  0000 0000     FF  1111 1111

7007    00  0000 0000     FF  1111 1111

7008    00  0000 0000     FF  1111 1111

7009    00  0000 0000     FF  1111 1111

700A    00  0000 0000     FF  1111 1111

700B    00  0000 0000     FF  1111 1111

700C    00  0000 0000     FF  1111 1111

700D    00  0000 0000     FF  1111 1111

700E    00  0000 0000     FF  1111 1111

700F    00  0000 0000     FF  1111 1111

7010    00  0000 0000     FF  1111 1111

7011    00  0000 0000     FF  1111 1111

"CR" to continue test - "SP", "^C" to quit, "^X" to restart.

Internal Prompts

----------------

The program prompts for the starting and ending addresses of

the block of memory to be tested and offers default values  which

will  test the largest possible block of memory.  The default top

of  the block to be tested depends on whether the -B  option  was

specified.  If  the default values are chosen,  the actual  value

determined  by the program is displayed next to the prompt as  if

the user had entered the value.

When specific values are entered by the user a valid 4 digit

hexadecimal  address  must be given or an error message  will  be

displayed and the prompt issued again.

The program may be aborted at any time by hitting any of the

keys specified in the abort prompt.

The program prompts as follows:-

MEMRS Memory Test - Version 1.9

BDOS located at D300     Top of memory at FFFF

Press "SP" or "^C" to warm boot at any time

or "^X" to restart test from beginning.

Response of "CR" gives default test range

which starts above this program and ends at BDOS

Starting address (Hex or "CR") 08F7

Ending address   (Hex or "CR") D3F9

"CR" to continue test - "SP", "^C" to quit, "^X" to restart.

Tutorial on Memory Testing

--------------------------

There  is no practical way to do a thorough test on all of a

computers memory in one pass. The memory test program itself will

occupy  some  memory  which it  cannot  thoroughly  test.  It  is

possible  to write simple memory tests which are able to test the

memory  that the reside in but these tests will not  detect  many

types of memory errors.

Memory  faults can be fairly simple gross errors where a bit

is permanently stuck (high or low) or they may be  timing/pattern

sensitive  where  the  fault  is  only  evident  with  a  certain

combination  of  data  in surrounding bits.  To attempt  to  test

memory as thoroughly as possible several different types of tests

need to be performed.  MEMRS has a wide variety of tests and  has

proven   particularly   effective  in  locating  bad  memory   in

comparison  to  large number of other memory test  programs  with

which  it has been compared.  It does not perform any  long  term

memory retention testing or any "burn in" testing,  however these

types  of tests are best performed over long periods of time  and

require  highly  optimised programs to execute in realistic  time

spans.

MEMRS  is intended as a relatively fast but thorough test to

quickly  establish whether any of the more common  memory  errors

has  occured.   In  practical terms,  the whole of  a  computer's

memory  rarely  fails simultaneously.  Memory failures  generally

occur in individual memory chips which usually cover no more than

16K of memory space (though this will change as 64K chips  become

readily available).  In this case it is usually possible in  CP/M

systems  to readdress memory so that the suspected bad memory  is

located  in  an area that can be tested with MEMRS.  If  the  bad

memory  is  located in any of the CP/M system areas it is  highly

unlikely  that you will be able to reliably load any programs  at

all from disk so there is not a great deal of point in  designing

memory tests for these addresses.

If  an error is suspected in the CP/M BIOS memory  area  the

best  move  is to build a smaller size CP/M system that does  not

use  the  suspected bad memory and the load MEMRS  and  test  the

suspect  memory.  MEMRS will test any address explicitly given to

it. It does not test to see if you will kill CP/M or itself.

The  random  number  only test is  useful  for  longer  term

testing of memory.  If an intermittent fault is suspected and the

memory  has  passed  several "full" test cycles  it  is  probably

worthwhile stopping the test and restarting it as a random number

only test which will cycle much faster.

The  normal default ending address of the bottom of the BIOS

gives the largest possible block of memory to test. It also means

that all console input and output from the program goes  directly

to the BIOS jump table so it is not possible to log the output on

the  printer  using  Control P.  Specifying the  BDOS  option  at

invokation  directs all console output via the BDOS  which  means

that  it  is  possible to log the test output on the  printer  by

typing Control P before the program is invoked.

END OF MEMRS.DOC

#######################